# CPE 325: Intro to Embedded Computer System

Lab05

Subroutines, passing parameters, Hardware multiplier

Submitted by: Nolan Anderson

Date of Experiment: 09/30/2020

Report Deadline: 09/30/2020

# Introduction

This lab covers subroutines, passing parameters, and hardware multiplication. We use subroutines to make our code more readable and simpler than writing everything out manually and essentially “hard” coding it in. These subroutines, in different files, makes changes occur much faster and more straight forward, and also helps to eliminate confusion on what you are working on.

Passing parameters is very important in this lab because we do not want to overwrite anything we are doing in our subroutines. Using the stack helps us do this where we push and “pop” (really moving) the values to and from the stack. This is especially important in the hardware multiplication.

Hardware multiplication is using the actual instructions built in to do multiplication instead of implementing and algorithm such as shift add multiply. As you can see, using the specific and general purpose registers makes the code much simpler and faster than doing it with an algorithm.

# Theory

Write short notes on each topic discussed in lab.:

Subroutines:

Subroutines help to simplify code and perform specific tasks. They can take in parameters and you go to them with the CALL instruction. You can get out of a subroutine with the RETURN call. Subroutines are very helpful in making your code simpler and more readable.

Passing Parameters (3 different ways):

When passing parameters, you can either add it to the stack and pull it from there later on or you can store the values into registers or in memory locations. Personally, I think the best way to store the data is to put it into specific spots in memory because it allows you to not overwrite any registers or mess with the stack too much. It does require a little more coding but I believe it would be a little more robust.

Hardware Multiplier:

The hardware multiplier, unlike multiplying using software, uses the general-purpose registers to multiply data together. It is simpler than something like the software multiplier that we implemented here in this lab. The run time is also significantly lower. It can be done up to 16-bit multiplication and the register you move it to determines the type of multiplication that will be performed.

Results & Observation

Copy the question from the assignment here:

1. How do you pass parameters to a subroutine using stack? Explain how you extract

parameters that you pass using this technique.

- I pushed the values onto the stack using push #swarr etc. and then in the calc_power.asm file, I moved the position on the stack pointer to the register that I wanted to work with.

## Results Screenshots/Pictures:

- Using b = 3

- Hardware Multiplier:

- Software Multiplier:

## 3. Measure the number of clock cycles used by each subroutine for a small range of values.

## Comment of the efficiency of each subroutine.

I have inserted images of my calculation for the number of clock cycles each instruction takes. As you can see, the hardware multiplier is over 2 times faster than the software multiplier. Total is the number of clock cycles at the end.

Calc power efficiency:					Hardware Multiplication efficiency:

Software Multiplication efficiency:

## Observations:

Hardware implementations are much easier to code and use. I am very interested to see if we keep doing something like hardware multipliers because it is honestly really cool. We do enough software in my opinion and using hardware to our advantage ( and to its advantage ) seems to be a much better use of our time.

Conclusion

Write your conclusion. (Explain what you have learnt and issues you faced)

Folder Link:

https://drive.google.com/drive/folders/1_Y3ABMDhCUxc9phtQ8JKHCM8LDOK7k7J?usp=sharing

Video link:

https://drive.google.com/file/d/1DmtWEoQLX_pTyNtjfBN_lhFrb2UboLKI/view?usp=sharing

Appendix on the next page

Appendix

Appendix 1 – main.asm

;------------------------------------------------------------------------------- ; File : main.asm ; Function : Push data onto the stack and call a function in another file ; Description: This file creates an array of 10 bytes for a hardware and software ; multiplier ; Input : A b value to be multiplied ; Output : No output, see registers and memory browser. ; Author : N. Anderson npa0002@uah.edu ; Date : 09/30/2020 ;------------------------------------------------------------------------------- .cdecls C,LIST,"msp430.h" ; Include device header file ;------------------------------------------------------------------------------- .def RESET ; Export program entry-point to ; make it known to linker. .ref calc_power ;------------------------------------------------------------------------------- ; Allocation for result .bss swarr, 10 ; 10 bytes for software array .bss hwarr, 10 ; 10 bytes for hardware array .data b: .int 3 ; int variable to intput into the code. result: .int ; result variable ;------------------------------------------------------------------------------- .text ; Assemble into program memory. .retain ; Override ELF conditional linking ; and retain current section. .retainrefs ; And retain any sections that have ; references to current section. ;------------------------------------------------------------------------------- RESET mov.w #__STACK_END,SP ; Initialize stackpointer StopWDT mov.w #WDTPW¦WDTHOLD,&WDTCTL ; Stop watchdog timer ;------------------------------------------------------------------------------- ; Main loop here ;------------------------------------------------------------------------------- main: push #swarr ; Push the software array address onto the stack. push #hwarr ; Push the hardware array onto the stack. push result ; Push the result onto the stack. mov b, R4 ; Mov b into R4. mov #5, R8 ; Counter for the loops in calc_power. call #calc_power ; A call to the calc_power.asm file to start the loops. jmp $ ; jump infinetly. lend: nop ;------------------------------------------------------------------------------- ; Stack Pointer definition ;------------------------------------------------------------------------------- .global __STACK_END .sect .stack ;------------------------------------------------------------------------------- ; Interrupt Vectors ;------------------------------------------------------------------------------- .sect ".reset" ; MSP430 RESET Vector .short RESET

Appendix 2 – calc_power.asm

;------------------------------------------------------------------------------- ; File : calc_power.asm ; Function : This file takes the data off of the stack, stores it into registers ; and uses those register values to perform operations ; Description: calc_power takes the data off the stack and initializes different data. ; Input : Input comes from main.asm which is really just the stack data. ; Output : Output is in the memory browser. ; Author : N. Anderson npa0002@uah.edu ; Date : 09/30/2020 ;------------------------------------------------------------------------------- .cdecls C,LIST,"msp430.h" ; Include device header file .def calc_power ; Define the calc_power function. .ref SW_Mult ; Reference to the Software multiplier file. .ref HW_Mult ; Reference to the Hardware multiplier file. .text calc_power: mov 6(SP), R5 ; Move the Software array address into R5 mov 4(SP), R6 ; Move the Hardware array address into R6 mov 2(SP), R7 ; Move the result into R7 mov #1, R12 ; R12 will get 1 for the first iteration of the software loop. jmp SWLoop ; Jump to the SWLoop, it will carry out from there. SWLoop: call #SW_Mult ; Call the software multiplier. incd R5 ; Increment R5 to the next index. dec R8 ; Decrement R8 (counter). jnz SWLoop ; Jump not zero back to software loop. mov #5, R8 ; reset power counter HWLoop: call #HW_Mult ; Call the hardware muliplier. incd R6 ; Increment to the next index. dec R8 ; Decrement the counter. jnz HWLoop ; Jump not zero back to hardware loop. lend: nop

Appendix 3 – SW_Mult.asm

;------------------------------------------------------------------------------- ; File : SW_Mult.asm ; Function : This files function is to perform software multiplication on a value ; Description: This file takes in R7 and R4 and uses shift - add - multiplication ; to find the powers of 4 from 1-5 ; Input : R4 and R7 from main and calc_power.asm ; Output : See memory browser ; Author : N. Anderson npa0002@uah.edu ; Date : 09/30/2020 ;------------------------------------------------------------------------------- .cdecls C,LIST,"msp430.h" ; Include device header file .def SW_Mult .text SW_Mult: mov #16, R9 ; Initializing a bit counter. mov #0, R10 ; Initialize the result number to R10. mov R4, R11 ; Move b into R11 so that we can perform operations w/o changing its value jmp bitcheck ; Jump straight to bitcheck. bitcheck: mov R12, R13 ; Mov b into R13 so that we can perform operations without changing its value and #0x01, R13 ; bitchecking to see if the LSB is 1 cmp #0x01, R13 ; Cmopare operation for previous AND instruction jne noadd ; If it is not equal (i.e. = 0) then skip the add instruction. add R11, R10 ; Add to the running total jmp noadd ; Go straight to s noadd: rrc R12 ; Shift B to right by 1. rla R11 ; Shift A left by 1. dec R9 ; Decrement the bit counter. jnz bitcheck ; If bit counter is not zero, go to next bit to add. mov R12, R13 ; If it is 0, move the result into R13. and #0x10, R13 ; Check to see if it is a negative number cmp #0x10, R13 ; If it is negative, jump to subtract from A from result. jeq neg ; Jump if equal to negative jmp end ; Unconditionally jump to end neg: sub R11, R10 ; Sub instruction for negative numbers jmp end ; Unconditionally jump to end. end: mov R10, 0(R5) ; Move the result into the next part of the mov R10, R12 ; Put the result into B for the next power operation. ret ; Return back to calc_power.asm

Appendix 4 – HW_Mult.asm

;------------------------------------------------------------------------------- ; File : HW_mult.asm ; Function : To perform a hardware multiplication on predefined values ; Description: This file will take in values and use hardware multiplication to ; find the powers from 1-5 ; Input : R4 and R7 from main and calc_power.asm ; Output : See memory browser ; Author : N. Anderson npa0002@uah.edu ; Date : 09/30/2020 ;------------------------------------------------------------------------------- .cdecls C,LIST,"msp430.h" ; Include device header file .def HW_Mult .text HW_Mult: mov R4, &MPY ; move R4 into the unsinged 16 bit multiplication register mov R7, &OP2 ; move R4 into the general purpose operator. multiply by... nop nop nop mov RESLO, R7 ; move the result the R7. mov R7, 0(R6) ; move result into R6 ret